Name of Layer:  Topographic Data

Names of File: ALLTOPO  (zipped in TOPO.ZIP)

Description:  Contour lines at 2- and 5-foot intervals for County area outside Raleigh; includes index contours and depressions, intermediate contours and depressions, labels on index contours, spot heights, and water elevations.

Area of Coverage:  County-wide except Raleigh’s jurisdiction

Distribution Format: Shape file

Conversion Method: All data were compiled from aerial photography, ground surveys, and fully analytical aerotriangulation.  Dates of photography vary by location. Data in and around the towns of Apex and Garner were compiled from photography flown in 1996;  Cary data were compiled in 1990 from photography flown in 1988-89; all other data were compiled from photography flown in 1981, which was scanned, vectorized, reprojected into NAD 83 and incorporated into the County’s GIS in 1991.

Update Method/Cycle: new county-wide topo planned for completion in 2001

Accuracy: National Map Accuracy Standards for 100 scale mapping as outlined in the "Technical Specifications for Base, Cadastral, and Digital Mapping," dated November 1987 (NC Land Records Management Program).

Currency: see flight dates

Projection Information:  NC State Plane, Zone 3200; NAD 83 (reprojected from NAD 27); Units:  Feet

Data Types(s): Arcs

Responsible Agency:  Wake County GIS

Contact Information: Carter Vickery (919) 856-6212  carter.vickery@co.wake.nc.us

Attribute Definitions: The topographic data includes index contours and depressions. The intermediate contours and depressions occur at the elevations that are multiples of two (2) excluding the index contours and depressions. Both the index and intermediate contours can be either visible, obscured or hidden, or a combination of the last two. The depression lines indicate areas that close on themselves because the area they surround have a lower elevation.

Topographic data includes both index and intermediate contours at intervals of 2- and 5-feet.  Every fifth contour is an index contour.  For areas with 2-foot intervals, index contours occur at multiples of 10.  Areas with 5-foot contour intervals have index contours on multiples of 0, 25, 50, and 75 feet.  Both index and intermediate contours can either be visible or obscured or hidden or a combination of obscured and hidden.  Obscured lines are areas where the ground was obscured on the source photography and the contours are interpolated from surrounding points, and are indicated by dashed lines.  Depressions are areas of terrain that are lower than their surroundings.

Elevation:  elevation in feet

Index contour: whether arc represents an index contour (Y/N)

Depression:  Whether arc represents a depression Y/N

Visible: ground visible from photography or obscured (Y/N)